Internal memo no. 912-178 - const div					Date: 13-11-25

Subject: cleaning of finished granite/tiles with Aquatak machine.

Keywords: Aquatak machine, cleaning.

- Very large amounts are being spent unnecessarily for cleaning of finished surfaces.

- This job can easily be done using a high pressure washer of 120 bar and above.

- A standard operating procedure  for use of Bosch Easy Aquatak120 is given under.

  - Machine cost is about Rs. 13,000/-

  - Purchase a PVC drum of about 50 to 60 ltrs capacity.

  - Fix a zoloto gun metal ball valve at its bottom with a nozzle.

  - Connect the Aquatak machine to the PVC drum with a 20mm braded hose pipe of 10 to 15mtrs length. Ensure that a hose pipe clip is attached at both ends.

  - Connect a 30mtr – 2.5 sqmm  (3/20) alumnium service wire to the machine with a male and female industrial plug on either ends.

  - The drum can be placed one flight above or just 200mm above the machine level.

  - The machine has 2 nozzles. Do not use the rotary nozzle. Only use the fixed fan jet nozzle.

  - The fan jet nozzle can be rotated to produce a point jet or a fan jet.

  - Use point jet for heavy cleaning including motor. Note that if overused – grouting will get removed. The jet can easily scratch/make holes or remove cement plaster.

  - Most of the times use a fan jet for cleaning.

  - Use a machine for not more than 30 minutes at a time. 10 minutes rest has to be given for the machine to cool down.

- Machine can be bought back by MHTR at 60% of the original cost and resold to any site at 65% of the original cost.

- Practice of paying Rs.1 to 4 per sft for cleaning to be stopped. Instead employ house keeping for cleaning. Alternately, Rs. 0.25 to 0.50 may be paid to any contractor for cleaning tiles/granite on floor, walls, staircases, toilets, etc.

- Photographs of the machine is attached.

Soham Modi.