Circular No. 844(G) - construction						Date: 12.11.2025

Sub.: Guideline rates for painting works.

Keywords: Painting, Minimum stock.

- The system of awarding contracts for painting is being revised for the following reasons:

  - Painters are using duplicate material.

  - Vendors are supplying duplicate material – unknown to painter.

  - Unauthorized brands are being used.

  - Paints are not being used as per guidelines of manufacturer.

  - Painters are diluting the paint to reduce cost.

- Accordingly, the entire SOP is being revised as under:

  - Rates are based on cost of paint, coverage, labour cost, profit and cost of tools.

  - Coverage is specified by manufacturer.

  - Paint will be supplied by MHTR.

  - Engineers shall ensure that at the time of billing the quantity of paint supplied matches the quantity specified by the manufacturer (coverage). It must be within +/- 15% of coverage specified.

  - Every requisition raised for material must clearly specify the painter/contractor.

  - MHTR shall raise the bills in favour of the respective project, however, the contractor who is using the material should be clearly specified in the bill. Accountants to pay to MHTR and debit to respective contractor.

  - Contractors can raise the bill for the balance amount. However, for raising running bills the contractors can raise bills part wise at labour rate + 15%.

  - Contractors may  raise non-GST bills or GST bills based on their revenues. If the revenue exceed Rs. 20 lakhs per year, then they should be strongly advised to raise GST bills.

- Rates for painting works have been rationalized based on coverage. The new rates which shall be applicable from immediate effect including Vivopolis, AMTZ4554, MCMET and other new works.

- Rs. 1/- per sft is being paid for primer as labour charges, this is inclusive of surface preparations, scraping, etc. additionally Rs. 1/- is being paid for I and II coats each. These labour rates are inclusive of all tools, rope (jula), spider kit, ladders, etc. In case of texture or inaccessible areas scaffolding to be provided at developers cost.

- The rates give under are for specific brands. MHTR shall be free to supply alternate brands – however, the cost to be deducted from the painter shall be based on rates given herein.

- The rates given below are inclusive of GST.

- The details of coverage are given in Annexure -I.

- The details of rates are given in Annexure -II.

- Details of lumsum rates for interior works of flats and villas is given in Annexure -III.

- A detailed excel sheet of calculation is attached herein as Annexure – IV & V.

- Clarification - This is not a labour contract. The contractors are being given 12% profit + 5% towards tools/etc., even on the material cost. Therefore, say the rate of primer + 2 coats of paint is Rs. 10/- per sft inclusive of GST and the paint consumed is say Rs. 6.25 (billed to the project), then the contractor has to raise the bill for the balance Rs. 3.75 (plus GST – optional).

- Notes:

- For dark colours in plastic emulsion, ACE or APEX painting Project Managers may add Rs. 1/- to Rs. 2/- per sft on the above rates depending on difficulty of work and darkness of colour.

- Rates inclusive of all material unless otherwise specified.

- Branded material should be used for all materials and restrict to the brands specified herein:

  - Primer – Asian

  - Tractor Emulsion - smooth – Asian.

  - Exterior emulsion – Apex ultima of Asian. In some cases Ace and Apex have been used. Ace or Apex can be used for compound wall.

  - Interior plastic emulsion – Royale by Asian

  - Luppum – Altek.

  - Cement putty – Birla wall care putty or equivalent.

  - Red-oxide – Asian.

  - Enamel – Asian Premium Apcolite.

  - For curb stone use Indigo floor paint – green and yellow  colour.

- Purchase shall ensure that there is an alternate brand for these paints. First preference is Birla Opus and second preference is Berger. Annexure -V to be added to this circular specifying minimum stock of items to be maintained by MHTR. When non-standard colours are being used MHTR may directly procure from vendor and supply to site.

- Project managers shall ensure that only the specified brands are used.  In case of any change authorization in writing or email must be obtained from MD.

ANNEXURE -III

Lumsum rates for interior rates for flats and villas

| Sl. No | Description | Units | Rate |
| --- | --- | --- | --- |
|  | Turnkey job for painting inside flat with luppum, primer, OBD, Enamel, POP gap filling, crack filling, washing & cleaning including doors, door frames, beading, black japan, lofts, etc. Excludes balcony painting, grills, templates and main door polishing. Minor additions & alterations included. | Sft of super built up area as mentioned in brochure | 32.00 |
|  | Turnkey job for painting bungalow / town house with luppum, primer, Tractor Emulsion, Enamel, Exterior Emulsion, POP gap filling, crack filling, washing & cleaning including doors, door frames, beading, black japan, lofts, external painting, compound wall, etc. Excludes grills, railings, gates and main door polish. Minor additions & alterations included. | Sft of super built up area as mentioned in brochure | 48.00 for villas in city. Pay 5/- per sft extra for AGH |